Name: Prathamesh Sai                                                       Student Number: 19314123

QUESTION 1. Connect the circuit below using Multisim placing the voltmeter and ammeters at the locations shown. The supply voltage is:

vt= 20 Sin 1000πt +π4V

(i) Run the simulation. What do you observe? Please provide a superimposed plot

of the voltages and currents using Grapher. [10 Marks]

My Circuit:

My graph:

My observations:

- The maximum voltage of PR2 is 19.970V, instead of the 20V supply voltage. This could be due to a percentage error in Multisim.

- The minimum voltage of PR2 is -19.970V.

- The maximum current of PR1 and PR3 is 3.1573 A, and the minimum current is -3.1573 A.

- The maximum current of PR4 is 199.70 mA and the minimum current is#

-199.70mA.

(ii) Calculate the current through the capacitor. Verify your result with the reading

from the plot. Why is the supply voltage 90 degrees out of phase with the

capacitor current? [20 Marks]

I = C ×dVdt

I = (50x10-6 )  ×  (d(20 Sin (1000πt + π4) dt V

t = 343.77ms for PR3

I = 50x10-620000πcos1000πt+π4

I = 50x10-620000πcos1000π343.77×10-3+π4

I = 50x10-66270.786876

I = 3.135393438 Amps

This stands to be true with out plot, with a small percentage error. Our plot illustrates 3.1573 Amps but our calculations give us 3.135393438 Amps.

Why is it out of phase?

- When the change in voltage with respect to time, dvtdt=0, this happens at Vmax. When this is true, we can see from the plot that I = 0 at this point.

- The change of current with respect to time, I(t), is generated by dv(t)/d(t), the change in voltage with respect to time.

- When dv(t)/dt = 0

- v(t) = Vmax

- i(t) = 0

- This is a 90° difference between v(t) and i(t). This explains why the supply voltage is 90° out of phase with the capacitor current.

To prove this mathematically,

I(t) = C ×dvtdt

Multiply both sides by Sin

LHS = Sin × I(t) 	                                                 RHS = Sin[ C × dv(t)/d(t) ]

Now if we differentiate with respect to I(t),

LHS = Still contains Sin.                  RHS= Sin changes to Cos through differentiation.

Sin and Cos are 90° out of phase with each other. This further proves our point.

(iii) Calculate the total current drawn from the source. Compare with the result given

by the simulator. [10 Marks]

The total current drawn from the source = current from capacitor + current from resistor

I (total) = 3.135393438A + current from resistor

Current from resistor = V/R = 20Sin1000πt +π4100 = 0.2Sin1000πt +π4

I(total) = 3.1573A + 0.2Sin1000πt +π4

We can use t = 343.77 x10-3 from Multisim.

I total= 3.1573 + 0.2Sin1000π343.77×10-3+π4

I total= 3.169858104 A

QUESTION 2. Connect up online the circuit below using Multisim:

Place voltmeters and an ammeter at the locations indicated. The supply current is:

i(t) = 5 Sin (1000 π t + π /3) A

Run the simulation.

(i) Calculate the voltage across the inductor. Verify your result by observing this

voltage on Grapher. Why is the inductor voltage out of phase by approximately

90 degrees with the load (or supply) voltage?

[25 Marks]

My circuit:

V = L ×dIdt

V = 10X10-3×d(5Sin(1000 πt + π3)dt

V = (10X10-3)× (5000πCos1000πt+π3

t =  6.5261 seconds

V = (10X10-3(5000πCos1000π6.5261+π3

V = 32.65869205 V

Observations:

- Through our calculations, we get 32.65869205 V, but Multisim gives us two values for Voltage when t = 6.5261.

V = 11.853 V                                         V = 42.478 V

- Hence, the voltage range (Vrange) = [11.853 –> 42.475] when t = 6.5261.

- Since our value of 32.65869205 V is between the range given by Multisim, we can conclude that our calculations were correct.

Why is it out of phase?

- When the rate of change of PR1 (= dv(t)/dt) is equal to zero, this happens at Vmax. At this point, PR3 = 0 (i.e I = 0). This means that the voltage (V) is 90° out of phase with the current (I).

- Since V ∝ I, the Inductor voltage is 90° out of phase with the supply voltage.

To prove this mathematically,

V = L ×dIdt

Multiply both sides by Sin

LHS = Sin × V                                                           RHS = Sin[ L × dIdt ]

Now if we differentiate with respect to time to get dv(t)

LHS = Still contains Sin.	          RHS= Sin changes to Cos through differentiation.

Sin and Cos are 90° out of phase with each other. Hence the voltage (V) across the inductor is 90° out of phase with the current (I).

- However, since V ∝ I, the voltage across the Inductor is 90° out of phase with the supply voltage.

This further proves our point. This was similar to Question 1 part (ii).

(ii) Calculate the total voltage across the load (resistor-inductor combination). Verify

(approximately) your result by observing this voltage on Grapher.

[15 Marks]

- We already have the voltage across the inductor from part (i), so we only need to calculate the voltage of the resistor.

- To find the voltage across the resistor, we use:

V = IR

Where,                 I= 5 Sin 1000 πt + π3

R = 100Ω

V = 5 Sin (1000 πt + π /3) x (100Ω)

V = 500 Sin(1000 πt + π /3)

t = 6.5261 seconds

V = 489.0738004 volts

This value is approximately equal to the value given by Multisim for the voltage across PR1 (the resistor which is 524.06V.

Voltage across inductor  +  voltage across resistor  = total voltage across the load

32.65869205 V          +       489.0738004 V             =                  521.7324925 V

- This is the value of the total voltage across the load (resistor-inductor combination)

- We can verify this result approximately with Multisim.

- Multisim calculates 524.06 V

- Our calculations calculate 521.7324925 V

This verifies our calculations for part (ii) and hence proves our point.